# CANomaly-LSTM

## Scenario Purpose and Scope

The purpose of this scenario is to learn normal behavior in in-vehicle CAN-Bus traffic using deep learning (LSTM-Autoencoder) and to detect anomalies such as fake message injection, replay attacks, unauthorized device access, and data corruption. Scope includes time-series feature extraction from CAN frames, model training, real-time scoring and alarm generation.

## Summary

The model is trained only with normal CAN message windows and attempts to reconstruct those windows. A real-time incoming window is scored by the model's reconstruction error; if the error exceeds a threshold the window is considered anomalous. This enables proactive detection of spoofed, replayed, or unexpected ID/payload pattern messages.

## Target Assets

- CAN-Bus communication network (time-series message stream)
- Critical inter-ECU signals (brake, steering, BMS, etc.)
- In-vehicle Intrusion Detection System (IDS) / Incident Management component

## Relevant Vulnerabilities

- CAN-Bus lacks default authentication, encryption and integrity controls.
- Insufficient segmentation between critical and non-critical networks (gateway vulnerabilities).
- Rule-based systems cannot generalize to novel/complex attack patterns.

## Threat Category (STRIDE)

- Spoofing: Sending forged messages while acting as a legitimate ECU.
- Tampering: Manipulating normal data flow (payload/ID/frequency).
- Denial of Service (DoS) – secondary: flooding the bus with fake messages at high frequency.

## Attack Vector and Steps

Vector: Access to CAN via telematics/infotainment vulnerabilities, malicious OBD-II devices, or weak gateway.

Step 1 – Reconnaissance: The attacker passively listens to traffic to learn IDs, DLC and payload patterns.

Step 2 – Manipulation:
  • Spoof: Broadcast forged commands using legitimate IDs (e.g., brake).
  • Replay: Re-broadcast previous valid messages at higher frequency.
  • Unauthorized device: Generate traffic from IDs that are not normally seen.
  • Corruption: Alter payload bytes with abnormal patterns.

Step 3 – Outcome: An ECU/VCU may execute incorrect actions (sudden braking, incorrect indicators, misleading BMS).

## Detection Methods (Deep Learning Approach)

- Features: CAN ID (one-hot/embedding), payload bytes b0–b7, inter-arrival time (IAT), rolling statistics per ID and delta bytes.
- Windowing: Sliding window (e.g., 50–100 messages) to capture temporal context.
- Model: LSTM-Autoencoder trained only on normal windows using MSE loss.
- Scoring: Window MSE → anomaly score; threshold chosen via ROC/F1 or using the 99–99.9 percentile of normal error.
- Alarm: Score > threshold ⇒ event logged + alert; latency is measured.

## Impact and Outcomes

- Early warning: Spoof/replay/unauthorized attacks can be detected before commands are executed.
- False-positive control: Thresholding and feature engineering keep FP low.
- Robustness: More general detection compared to rule-based systems for unseen patterns.

## Metrics and Performance Criteria (example)

- Overall accuracy: 99% (example study).
- For anomaly class: Precision ≈ 98%, Recall ≈ 50%, F1 ≈ 66% (threshold at F1 max).
- Improvement target: Choose threshold to favor recall, use window=100 and add delta/rolling features to target Recall ≥ 80%.
- Latency: Alarm <200 ms from anomaly start (depends on deployment/device).

## Countermeasures / Complementary Controls

- Apply MAC/signature and integrity verification over CAN where possible.
- Network segmentation and gateway rate/flow limiting.
- Use a hybrid approach: model-based detection plus statistical rules and signature-based IDS.